# FIND_SHEETS utility

FIND_SHEETS program is PCAD2001 DBX (schematic) utility, which gives simple, elegant and human readably way of showing connections for SHEET CONNECTORS (in the rest refe-renced as SC). In addition to standard SC, utility manages also BUS SCHEET CONNECTORS (BUS SC). For explanation BUS SC see in the rest of the document.

SEET CONNECTOR is special PCAD component (without physical presentation in PCB) which should provide good (visible) presentation for connection for particular NET with all nodes at various SHEETS inside one project and simple signal following. Unfortunately, embedded PCAD2001’s mode of presenting SC practically is “useless”, due to lot of informations (SHEET NAME, FIELD where is node located…) that occupy precious space at schematic.

FIND_SHEETS utility shows inter-sheet connections in simple way, only writing numbers of sheets, separated by commas. This way, selecting one low height font all informations can be located in small area.

Drawing below shows typical FIND_SHEETS’s ways of inter-sheets connection presentation.

Presenting in “ONE ROW”

RD*

@3,4,5,7,8

MWR*

@3,4,5,7,8

9,12

Presenting in “MORE ROWS”

Connections (assigned texts) can be presented in two ways:

- ONE ROW

- MORE ROWS

To provide this features (selecting one of more rows), SC must have attribute FIND_SHEET. By default, this attribute should be 0 (in this case all connections will be presented in “ONE ROW”). Setting this attribute to 1 (after next UPDATE), connections for this SC  will be presented in “MORE ROWS”.  This way is possible, for extremely long texts (generally in complex designs) provide good pre-sentation in more rows. SC without FIND_SHEET attribute behaves exactly as “ONE ROW” SC (can present connection in only one row).

Each SC must have unique referenced NET name (VIA PORT or WIRE NAME)

## BUS CONNECTORS

FIND_SHEETS utility supports also concept of BUS SHEET CONNECTORS (BUS SC). It mea-ns that is possible show inter-sheet connection also for BUSES. Drawing at next page shows typical BUS SCHEET CONNECTORS in this utility. There is not big differences respecting normal SC. Only difference is that BUS SCHEET CONNECTORS will show summary (reference) SHEETS for ALL signals that make one BUS. The same feature of “ONE ROW” or “MORE ROWS” presentation is valid also for BUS SC.

Presenting one BUS SC in ONE ROW

Presenting one BUS SC in MORE ROWS

AD0

@3,4,5,7,8

AD0-AD7

AD0

@3,4,5,7,8

12,15

AD0-AD7

## Some precautions for BUS SHEET CONNECTORS

To be recognized as BUS SHEET CONNECTOR (BUS SC) in FIND_SHEETS program, in addition to different name respecting normal SC (see below), must be satisfied also:

- BUS SC MUST be placed only at VERTICAL stub of any BUS (see above)

- Distance from BUS STUB to BUS SC origin must be <= 300 mils (or equivalent unit in mm or inch, if selected)

- BUS SC must be finished with some wire. Name of this wire is irrelevant (refe-rencing for BUS SC is not done by this name, but proximity of BUS STUB and signals in this BUS). Other solution is making standard BUS ENTRY to BUS and place for name one of signals which makes this BUS. In example above, we have BUS named AD0-AD7 and it is chosen signal AD0. This mode is recommended because better explains connection between BUS SC and BUS. We can add (only for better readability) text “– AD7” near to port AD and have “unique: name AD-AD7.

We can underline that signal name for BUS SC is irrelevant for FIND_SHEET utility.

## BUILDING SC and BUS SC

You must have SHEET CONNECTOR component (components) for using FIND_SHEETS utility.

Note that it must be built as true SHEET CONNECTOR component.

Graphic presentation for SYMBOL is irrelevant, but it is clear that SC (BUS SC) must satisfy:

- As soon as smaller drawing area, which will not occupy too space

- Have graphic that will show direction of signal (input, output or bi-directional)

Name (Type) and reference of normal SC is irrelevant, but BUS SC MUST have unique name (type) that starts with “SH_BUS “. It is only way for differencing normal SC and BUS SC. It addition, (noted above) FIND_SHEET attribute is highly recommended (for presenting conne-ctions in more rows)

This ZIP file contains also HIGHLY RECOMMANDED library sheet_conn.lib, which conta-ins READY MADE SC (BUS SC) connectors:

- SH_IN

- SH_OUT

- SH_BID

- SH_BUS_IN

- SH_BUS_OUT

- SH_BUS_BID

First three are for normal SC, and other three for BUS SC. Components are nearly the same. BUS SC are drawn with one centerline to better explain sense of BUS SC.

SHEET CONECTORS can be flipped as other components.

## USING FIND_SHEETS UTILITY

Picture above shows main menu of FIND_SHEETS utility

Commands from main menu (Update) are:

UPDATE: It will update (assign texts to all SC and BUS SC)

DELETE: It will delete all SC (BUS SC) assigned texts if you, due to any reason, need it.

UTILS: It will open new submenu with some local utils (see below)

STATUS: It will open new submenu with STATUS about number of SC (BUS SC), unassigned texts….

HELP: It is simple HELP about

UTILS submenu:

There is not any particular need for this submenu (picture below) and you will use it a little. Most of these functions you can achive directly in SCHEMATIC ENTRY. Anyway, these utilities will help you in better organization and finding particular SC (BUS SC) if you have some troubles.

Typical using will be when you want to change some ONE ROW SC (BUS SC ) to MORE ROWS after first UPDATE. It is possible simple selecting (or highlighting) these SC (BUS SC) in design and performing INSERT SELECTED (HIGHLIGHTED) in UTILS menu.

Picture above shows UTILS submenu of FIND_SHEETS utility

Commands from UTILS submenu are:

Highlight “one row”: It will HIGHLIGHT all SC (also BUS SC) that can be presented only in one row (including SC without FIND_SHEET attribute).

Highlight “more rows”: It will HIGHLIGHT all SC (also BUS SC) that can be presented only in more rows (obviously that it is valid only for SC with FIND_SHEET attribute set to 1).

Invert highlighted: It will invert value of SHEET_TYPE attribute (0 -> 1 and 1 -> 0) for highlighted SC (also BUS SC). Note that SC (BUS SC) without SHEET_TYPE attribute remain unchanged.

Invert selected: It will invert value of SHEET_TYPE attribute (0 -> 1 and  1 -> 0) for selected SC (also BUS SC). Note that SC (BUS SC) without SHEET_TYPE attribute remain unchanged.

Add attribute: It will add SHEET TYPE (set to 0) to all SC (also BUS SC). Without this attribute.

STATUS submenu:

This submenu is provided to show statistic about used SC (including BUS SC). Submenu contains the next information:

- ONE ROW SC: This FIELD will show total number of SC (also BUS SC) that will be presented in one row (it includes SC with FIELD_TYPE attribute set to 0 or SC without FIELD_TYPE attribute)

- MORE ROWS SC: This FIELD will show total number of SC (also BUS SC) that will be presented in more rows (it includes SC with FIELD_TYPE attribute set to 1)

- NO ST_T ATTRIBUTE: This FIELD will show SC (also BUS SC) without SHEET_TYPE attribute

- ASSIGNED TEXTS: This FIELD will show number of assigned texts to SC (BUS SC). Note that are possible situations when text is not assigned to SC (or BUS SC). There are two situation for this. First case is when NET assigned for SC (BUS SC) is not found at any other SHEET (node). It means that user probably has made some error in naming  NET for this SC (BUS SC). Second case is when there is not enough space to place text for this SC (BUS SC) and these texts will be assigned with “**”. In this case user should move this SC (BUS SC) more to SHEET center. It is also possible that there is not even enough space for place “**” text. This case will be signed with special message and empty text for this SC (BUS SC).

- TOO LONG TEXTS are number of texts assigned with “**”.

After UPDATE command user should perform STATUS command only for check. Normal case is that sum of “ONE ROW SC” and “MORE ROWS SC” should be equal to “ASSIGNED TEXTS”. Other two FIELDS (“NO SH_T ATTRIBUTE” and “TOO LONG TEXTS”) should be 0.

Commands from AUX menu (Settings):

This menu provides settings for some Place items (for assigned texts). Using these parameters user can control some text parameters. There are four possible parameters for selection:

Text type: User can select one of actual text types in design.

Left side start: This parameter will set start point for assigned texts at LEFT side of SCH, star-ting from SC (BUS SC) origin.

Right side start: This parameter will set start point for assigned texts at RIGHT side of SCH, starting from SC (BUS SC) origin.

Max row lenght: This parameter will set  maximal text length for MORE ROWS SC (BUS SC) and is irrelevant for ONE ROW SC (BUS SC).

NOTE: These parameters (except Text type) are set to default values (presented at picture) that should be OK for nearly all users. If someone wants to set other values, selection of new values must be followed by local command button  “UPDATE”. Note that these parameters are memo-rized  in schematic FILE (See under DESIGN INFO) and saved in FILE after command FILE SAVE.

Note that this setting is valid only in this FILE and opening some new design will set default values.

## RECOMMENDED PROCEDURE FOR USING  “FIND_SHEETS” UTILITY

- Create SC and BUS SC components in your system library (Highly recommended copy existing from library sheet_con.lib or built own, modifying them)

- Add SC (BUS SC) in design, following rules explained above (particularly for BUS SC) , leaving expected space for assigned texts at right and left side of SCH.

- You can perform FIND_SHEETS utility in any phase of SHEET building. It will help you to analyze connections. You should also use often STATUS submenu to check validity of SC (BUS SC).

- You will surely rearrange (in phase of SCH drawing) SC (BUS SC) text lengths and positions moving SC (BUS SC) or changing SHEET_TYPE attribute to show in ONE or MORE ROWS (1 -> 0 and 0 -> 1) and following eventually messages.

- Don’t forget that you can also DELETE all assigned texts and use embedded PCAD 2001 SHEET CONNECTOR features temporary or always in this design.

- After project is finished and before SCHEMATICS is archived, you could also manually move some assigned texts for improving positions and for better visual effect (Note that each new update after this will override these manually rearranged assigned texts).

- Don’t use @ symbol for starting your free texts in SCH. This symbol is reserved for FIND_SHEET utility.

It is highly recommended to name SHEETS starting from numbers. It will give you direct correlation between SHEETS and numbers of SHEET used in FIND_SHEET utility. For example, names of your SHEETS could be as follows:

1CPU

2XILINX_FPGA

3DIGITAL INPUT

.

.

You can minimize FIND_SHEETS program, and continue normal SCHEMATIC dra-wing. Some precaution must taken account. You should not insert new SHEETS when FIND_SHEETS program is minimized because number of SHEET and some other para-meters are set in start up procedure (when program starts). Better solution is anyway close FIND_SHEET after one session and open when there is need for new  (the same for other DBX utilities)

## NOTES

FIND_SHEET utility is tested with PCAD2001 (Service pack 2) and it is unknown prog-ram behavior for other versions.

- It is attached also example find_sheets_example.sch. This file (with 20 sheets) and numerous SC (BUS SC) contains all FIND_SHEETS’s features (SC and BUS SC connectors, one and more rows assigned text…). Note that this schematic electrically DOESN’T MAKE ANY SENSE and intention was only to show utility features (Real designs are much simpler).

- You should have also TABCTL32.OCX in your WINDOWS/SYSTEM dire-ctory for program execution (supposing that VB RUN and DBX32 DLL files already exist)

- It is attached also file sheet_conn.lib (above mentioned), which contains suggested SC and BUS SC.

Each comment is welcome at: semir@sitek.it