Aperture Loader Program

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

gap2eda.exe    Created 2/27/02    by John Bryant,     Ayrshire Electronics, LLC

This document contains information about the Aperture Loader program, which is used to prepare a PCAD2001 design file for importation of Gerber files.  Since PCAD2001 does not read imbedded Gerber apertures, it is necessary to pre-define any Dcodes used in Gerber data that is to be imported.

Gap2eda.exe is not a DBX utility, but rather operates on an ASCII save of your PCAD2001 design file, creating a new file with aperture Dcodes defined.

NOTE: This program requires an aperture file in the CAM350 “.gap” format as input.

Contents

--------------------------------------------------------------------------------

- Introduction to Aperture Loader

1.1 	Overview

  - Preparation

- Running Aperture Loader

1.0	Introduction to Aperture Loader

1.1  Overview

Aperture Loader was conceived with Reverse Engineering in mind, useful as part of the first steps in re-creating a pcb layout from only Gerber data.  Since the Gerber import function of PCAD was designed primarily to re-import Gerber data generated from a PCAD design and since all the Dcodes utilized within the Gerber files were defined prior to generation of the data, there was no need to extract Dcode definitions from the imported files.

But the ability to import Gerber data into a PCAD design can also be a wonderful tool for providing a background template for Reverse Engineering an existing circuit board when the only files available are Gerber.  Wonderful if only you didn’t have to manually enter all the Dcode definitions first.

Aperture Loader allows you to take an ASCII save of a PCAD2001 design file (without any apertures defined) and inject the aperture definitions from a CAM350 format .gap file.  Once this has been done, Gerber files corresponding to the .gap file may be successfully imported into the new version of the PCAD design created by Aperture Loader.

1.2  Preparation

As stated previously, Aperture Loader requires the use of one of the CAM350 family of Gerber editor programs, or some other capability of preparing a file of the apertures used in the Gerber data to be loaded, in the CAM350 .gap file format.

Assuming one has access to a CAM350 editor, step one is to load the Gerber files into that application.  Making sure the aperture definitions were successfully read from the Extended (RS274x) Gerber files OR an accompanying aperture file, export a “xxx.gap” file from CAM350.

NOTE: Aperture Loader only loads “Round”, “Rectangle”, “Square”, and “Oblong” aperture shapes.  All other shapes such as Thermals, Custom, etc. contained in the .gap file are ignored.

Step two is to open a blank (or “template”) design file in PCAD2001 and save it as ASCII. In actuality, the file may contain any amount of data, provided no apertures have been previously defined.

2.0  Running the Program

Start the Aperture Loader as you would any other standalone executable by double-clicking the gap2eda.exe file. Select your previously saved ASCII .pcb file using the drive, directory and file boxes located under “PCB File”.  Select the previously saved .gap file using the similar file boxes under “GAP File”.  The “Output File” window near the bottom of the form reflects the default name of the new PCAD design file to be created.  The name and directory path may be modified to the users choosing prior to pressing the “Start” button.

The program is initiated by pressing “Start”.  “Complete” will appear beneath the Status label when the program finishes.

You may now “Exit” Aperture Loader and open the new version of your PCAD2001 design file and import your Gerber data.

Enjoy !